Measurement and Measurement Error

PHYS 1313 S06

Prof. T.E. Coan

Version: 16 Jan ’06

Introduction

Physics makes both general and detailed statements about the physical universe and these statements are organized in such a way that they provide a model or a kind of coherent picture about how and why the universe works the way it does. These sets of statements are called “theories” and are much more than a simple list of “facts and figures” like you might find in an almanac or a telephone book (even though almanacs and telephone books are quite useful). A good physics theory is far more interested in principles than simple “facts.” Noting that the moon appears regularly in the night sky is far less interesting than understanding why it does so.

We have confidence that a particular physics theory is telling us something interesting about the physical universe because we are able to test quantitatively its predictions or statements about the universe. Indeed, all physics (and scientific) theories have this “put up or shut up” quality to them. For something to be called a physics “theory” in the first place, it must be falsifiable and therefore must make quantitative statements about the universe that can be then quantitatively tested. These tests are called “experiments.” The statement, “My girlfriend is the most charming woman in the world,” however true it may be, has no business being in a physics theory because it simply cannot be quantitatively tested.  If the experimental measurements are performed correctly and the observations are inconsistent with the theory, then at least some portion of the theory is wrong and needs to be modified. Yet, consistency between the theoretical predictions and the experimental observations never decisively proves a theory true. It is always possible that a more comprehensive theory can be developed which will also be consistent with these same experimental observations used to support the original theory. (For example, Einstein’s theory of relativity is a refinement of Newton’s theory of relativity and quantitatively describes phenomena that Newton did not know about.) Physicists, as well as other scientists, give a kind of provisional assent to their theories, however successfully they might agree with experimental observation. Comfortably working with uncertainty is a professional necessity for them.

Making careful quantitative measurements are important if we want to claim that a particular physics theory explains something about the world around us. Measurements represent some physical quantity. For example, 2.1 meter represents a distance, 7 kilograms represents a mass and 9.3 seconds represents a time. Notice that each of these quantities has a number (9.3) and a unit (“seconds”). The number tells you the amount and the unit tells you the thing that you are talking about. Both the number and the unit are required to specify a measured quantity. Leaving either out makes your answer wrong! Pay attention.

# Precision

There is a certain inherent inaccuracy or variation in any measurement we make in the laboratory. This inherent inaccuracy or variation is called experimental “error” and the word is not meant to imply incompetence on the part of the experimenter. Error merely reflects the condition that our measuring instruments are imperfect. This lack of perfection in our measuring procedure is to be contrasted with mistakes like adding two numbers incorrectly or incorrectly writing down a number from an instrument. Those mistakes have nothing to do with what I mean by experimental error and everything to do with the competence of the experimenter! Understanding and quantifying measurement error is important in experimental science because it is a measure of how seriously we should believe (or not believe) our theories abut how the physical universe works.

If I measure my mass to be 120.317 kilograms, that is a very precise measurement because it is very specific. It also happens to be a very inaccurate measurement because I am not quite that fat. My mass is considerably less, something like 85 kilograms. So, when we say that we have made a precise measurement we can also say that we have made a very specific measurement. When we say we have made an accurate measurement we are saying that our answer is close to the true value of the quantity. When we make measurements in the laboratory we should therefore distinguish between the precision and the accuracy of these measurements.

The so-called number of significant figures (“sig figs”) that a measurement contains indicates its the precision. In our mass example, the quantity 120.317 kilograms has 6 significant figures. This is rather precise as measurements go and is considerably more precise than anything you will ever measure in this course. Again, the fact that a measurement is precise does not make it accurate, just specific. In any event, it is useful to be familiar with how to recognize the number of significant figures in a number. The recipe is:

- The leftmost non-zero digit is the most significant digit;

- If there is no decimal point, the rightmost non-zero digit is the least significant;

- If there is a decimal point, the rightmost digit is the least significant, even if it is zero.

- All digits between the least significant and the most significant (inclusive) are themselves significant.

For example, the following four numbers have 4 significant figures:

2,314    2,314,000    2.314    9009    9.009     0.000009009    9.000

A small precaution is in order. Since a measurement has a number and a unit associated with it, you might wonder if you need to worry about the unit in determining how precise a measurement is. The answer is NO! The units have nothing to do with the precision of a measurement so just ignore them. For example 0.002 grams is NOT more precise than 12.1 kilograms. The first has 1 sig-fig and the second has 3 sig-figs.

No physical measurement is completely exact or even completely precise. The apparatus or the skill of the observer always limits accuracy and precision. For example, often physical measurements are made by reading a scale of some sort (ruler, thermometer, dial gauge, etc.). The fineness of the scale markings is limited and the width of the scale lines is greater than zero. In every case the final figure of the reading must be estimated and is therefore somewhat inaccurate. This last figure does indeed contain some useful information about the measured quantity, uncertain as it is. A significant figure is one that is reasonably trustworthy. One and only one estimated or uncertain figure should be retained and used in a measurement. This figure is the least significant figure. It is also called the “uncertainty” in your measurement.

For example, a length measurement of 2.500 meters (4 significant figures) is one digit less uncertain that a measurement of 2.50 meters (3 significant figures). If your measuring instrument is capable of 4 digit precision then you can distinguish between an item 2.500 meters long and one 2.501 meters long. If your instrument were precise to only 3 significant figures, then both items would appear to be 2.50 meters long and you could not distinguish between them. Again, infinite precision is not possible. All instruments have limits. It is important to know how much precision your instrument is capable of. For example, it would be silly to measure something with a toy ruler and then announce that the object was 9.3756283 centimeters long.

## Error

Perhaps you have had the experience of measuring something multiple times and not quite getting the same answer for each measurement. Your measured value could be a bit high or a bit low of the correct value. Perhaps you notice that the variations in your measurements have no fixed or predictable pattern. Your measured value could be as likely to be a bit too high as to be a bit to low. This variation in the measured quantity is called a random error. Indeed, with random errors an individual measurement of an object is just as likely to be a bit too high as to be a bit too low. For example, using a ruler often produces random errors because you will not consistently get the ruler’s zero line exactly aligned with the edge of the measured object every time you make a measurement.

Measurements may display good precision but yet be very inaccurate. Often, you may find that your measurements are usually consistently too high or too low from the accurate value. The measurements are “systematically” wrong. For example, weighing yourself with a scale that has a bad spring inside of it will yield weight measurements that are inaccurate, either too high or too low, even though the weight measurements may seem reasonable. You might, for example, notice that you seem to gain weight after Thanksgiving holiday and lose weight after being ill. These observations may in fact be true but the actual value for your weight will be inaccurate.  This is an example of a systematic error. Your measurements are systematically wrong. It is often very difficult to catch systematic errors and usually requires that you understand carefully the workings of your measuring device(s).

How do we write our answer, including its error, when we measure some quantity? We write it x=xbest±dx, where xbest is our best estimate of its main value and x is our estimate of its error. If you make a single measurement, xbest is the number you read off your measuring instrument, with a precision of half the smallest increment on your instrument’s gauge. For example, if your ruler has millimeter hash marks, then your xbset is the length to the nearest ½-millimeter (e.g., 61.0 mm, 66.5 mm, ...). The uncertainty x can be taken as half the smallest increment on the measuring scale. In our case, x = ½ millimeter.

But what do you do if you make multiple measurements of the same quantity to reduce random error (see below) and already know or can estimate the uncertainties in individual measurements, and are interested in the error of the final answer? For example, suppose you can measure lengths to a precision of +/- 3 mm. You measure the length of some quantity many times and now you want to make a statement about the uncertainty in your value of xbest. You might think that you ought to be able to make use of this knowledge of the uncertainty of an individual measurement. You are correct! To get the uncertainty in your final answer, you need to add the uncertainties in the individual measurements, but in a special way.  Specifically, you compute the sum

1s2=1s12,

where  si is the uncertainty in the ith individual measurement. In words, this formula says to add the squares of the reciprocals of the individual uncertainties. This may seem like a perverse way to proceed but it is actually quite sensible. If a particular measurement has a very large uncertainty (because you forgot to turn on part of the measuring device), and then you do not expect this particular measurement to be very useful for finding your final answer. Well, the above formula tells you this. If you take the reciprocal of a large number then you will get a small number. Adding a small number to some other number doesn’t always change the original number very much so in some cases you can just neglect the small number.

## Odds and Ends

Taking the mean (average) value for a set of measurements is a good way of reducing the effects of random errors and is the preferred way to get your value for xbest. As you increase the number of measurements, the mean value of these measurements will more closely resemble the actual value of the quantity you are trying to measure, assuming there are no systematic errors present. This is because you are just as likely to measure a value that is slightly too high as one that is too low, so that the random errors will “average themselves out.” The magnitude of the relative errors for the individual measurements gives you an idea of the uncertainty in your value.

Averaging your measurements will not help you when there are systematic errors present. In fact, there is no standard way to deal with systematic measurements other than changing your measurement technique. Often you do not know you have systematic errors or you have trouble estimating them, which is why they are so troublesome.

There is a final aspect of measurement that you should be aware of. We need to be specific about what exactly it is we are trying to measure. For example, in our weight example above, does it mean with or without clothes? At what time of day? Before or after a meal? You should be clear about the meaning of the quantity you are trying to measure so that people can accurately interpret the significance of your results.

## Summary Points

- The precision of a number is a measure of how specific it is.

- The accuracy of a number is how close to the truth it is. A measurement always has an error: random and/or systematic.

- Taking multiple measurements and averaging reduces random error.

- Changing your measuring technique reduces systematic error.

- Write your answer to a measurement as: x=xbest±dx unit.

Don’t forget the unit.

## Summary Questions

- What is the difference between accuracy and precision and why is this difference significant?

- How many significant figures are in 13600? 0.12040? 00045?

- What distinguishes a random error from a systematic error?

- What is the point of making multiple measurements of the same quantity before making a final statement about the numerical value of that quantity?

Objectives

- To see how measurements and error analysis are a fundamental part of experimental science.

- To make some actual measurements and analyze the errors in them.

- To observe and understand the difference between accuracy and precision.

- To understand the nature of random and systematic errors.

Equipment

Candle, ruler, “special” ruler, paper with closed curves, metal rod, material to be weighed (dry ice and alcohol) and triple beam balance.

Procedure

- Measure the length of the metal rods.

Using only the special ruler, measure the length of each piece 5 times. When you are finished, average the values to get a better measure of the piece’s true length. Next, use your plastic ruler to measure your pieces again. Measure 5 times as before and compute the average to refine your measured value. Make an “eyeball” estimate of your uncertainties.

- Measure the height of the candle flame.

Light your candle and let the flame burn steadily for a minute or so. Use the plastic ruler to measure the height of the flame. Make 10 measurements and try not to melt the ruler or your fingers. Hold the ruler a small distance away from the flame. Record your measurements. Estimate for each measurement its error si. Remember to write only a sensible number of significant figures.

- Measure your reaction time.

Your reaction time is the time that passes between some external stimulus and your first action. We will use an old method to measure your reaction time. A falling ruler will suffice. This is what to do.

- Have your partner hold the regular ruler vertically, holding it by the top and having the zero point toward the bottom.

- Place your thumb and forefinger at the ruler’s bottom, surrounding the zero point.  Be prepared to pinch the ruler as if it were to fall. Rest your forearm on the lab table to steady your hand.

- Your partner will drop the ruler without warning.

- Pinch and grab the falling ruler as fast as you can. Record the distance the ruler fell. This will tell you your reaction time.

- Compute your reaction time using Galileo’s formula: s = 12gt2. The meaning of the symbols will be explained in lab.

Make 5 measurements and record the corresponding reaction times. Record your reaction times on your data sheet. Do not mix your times with your partners. This means you will make 5 measurements per person. Compute and record the individual si’s for each measurement. Be sure that both you and your lab partner have your reaction times measured.

- Measure the diameter of a closed curve.

Measure the diameter of the large closed curve. Measure this curve diameter across 6 different diameters of the curve. Record your measurements. Compute the average diameter. Estimate for each measurement its si. Remember to write only a sensible number of significant figures.

- Measure the mass of a cold material.

Go to the instructor’s table with your partner, where you will be given a cup containing some alcohol and some crushed dry ice. Using the balance on the instructor’s table, measure and record the mass of the cup 6 times at 1 minute intervals. Warning: The dry ice and alcohol mixture is quite cold. If you stick your fingers in the mixture you will feel much pain. No one has yet been stupid enough to try and drink the mixture. Don’t be the first.

Measurement and Measurement Error

### PHYS 1313

Prof. T. E. Coan

Version: 16 Jan ‘06

Name: _____________________		Section: __________________

Abstract:

Analysis

1. Rod length

Rod 1		   Special Ruler	   si	           Plastic Ruler               si

Measurement 1    __________	_______	__________             _______

Measurement 2    __________	_______	__________             _______

Measurement 3    __________	_______	__________             _______

Measurement 4    __________	_______	__________             _______

Measurement 5    __________	_______	__________             _______

Measurement 6    __________	_______	__________             _______

Average Value     __________			__________

Uncertainty          __________			__________

- Explain the possible sources of error in this measurement?

- How well did your measurements with the special ruler agree with those done with the plastic ruler? If there was a disagreement, what kind of error was it? Random or systematic? What caused this error?

- Candle Flame

Flame height					          si

Measurement 1   ___________					___________

Measurement 2   ___________					___________

Measurement 3   ___________					___________

Measurement 4   ___________					___________

Measurement 5   ___________					___________

Measurement 6   ___________					___________

Measurement 7   ___________					___________

Measurement 8   ___________					___________

Measurement 9   ___________					___________

Measurement 10   ___________					___________

Average Value    ___________

Uncertainty         ___________

- Describe the possible sources of error in this measurement.

- What might you do to get a better measurement of the flame’s height?

- Your reaction time

Distance		            Time	            	si (time)

Measurement 1         ___________		________		________

Measurement 2         ___________		________		________

Measurement 3         ___________		________		________

Measurement 4         ___________		________		________

Measurement 5         ___________		________		________

Average value           ___________		________

Uncertainty               ___________		________

- Describe the possible sources of error in this measurement and explain their relevance.

4. Closed Curve Diameter

Curve diameter                              si

Measurement 1		___________________	_______________

Measurement 2		___________________	_______________

Measurement 3		___________________	_______________

Measurement 4		___________________	_______________

Measurement 5		___________________	_______________

Measurement 6		___________________	_______________

Average value			___________________	_______________

Uncertainty  			___________________

- Describe the possible sources of error in this measurement.

- What do the measurements tell you about the curve’s diameter?

- Mass of cold material.

Measurement 1	____________________

Measurement 2	____________________

Measurement 3	____________________

Measurement 4	____________________

Measurement 5	____________________

Measurement 6	____________________

- Describe the possible sources of error in this measurement and their relevance.

- Do you see any pattern in the measured masses? What is it?

- Which of your measurements (metal rods, curve diameter, flame height, etc.) was the most uncertain? Why?

- Which of your measurements (metal rods, curve diameter, flame height, etc.) was the least uncertain? Why?

- Which measurements (metal rods, curve diameter, flame height, etc.), if any, suffered from systematic error? Explain.

Conclusions

Succinctly describe what you learned today.